Name: TBD

Version 1.0 MVP

Introduction

This document is a guide to the user interface and functionality of Version 1.0 of the sizing software.

Logged in Navbar

- Left

  - Brand logo if applicable

  - Brand name if no logo

- Center

  - Application Navigation Menu

    - Projects dropdown menu

      - Create New Project

      - Project Library

    - Settings dropdown menu

      - Company information

      - Application defaults

      - Material Library

    - Resources

      - Help

        - Opens Help page

      - Building Code Resources

        - Opens Building Code Resources page

      - Terms of Service

        - Opens ToS page

      - TBD

    - Contact

      - Opens Contact page

- Right

  - Search Bar

    - Search whole application for information

  - Notification icon

    - Notification dropdown

  - Profile icon

    - Profile

      - Opens user profile information page

    - Subscription

      - Opens subscription settings page

    - Log out

      - Logs user out of application

User Dashboard (below navbar)

- Project Buttons Toolbar (top of dashboard)

  - New Project

    - Creates new project using default settings from user settings

  - Open Project

    - Opens project list window

  - Save Project

    - Saves current project to project list

  - Save As Project

    - Saves project as new project

  - Project Information

    - Opens project information window

  - Project Settings

    - Pulls default user settings

    - Can be modified for single project

- Member Type Toolbar (next to project buttons)

  - Joist

    - Opens new joist design

  - Beam

    - Opens new beam design

  - Rafter

    - Opens new rafter design

  - Roof Beam

    - Opens new roof beam design

  - Hip Beam

    - Opens new hip beam design

  - Column

    - Opens new column design

- Member viewer window (below project and toolbar buttons

  - Right click on member opens menu

    - Select Product

      - Opens material library for picking

    - Add new point load

      - Opens point load input window

    - Add new concentrated linear load

      - Opens concentrated linear load input window

    - Add new uniform load

      - Opens uniform load input window

    - Add new uniform linear load

      - Opens uniform linear load input window

    - Add new trapezoidal linear load

      - Opens trapezoidal linear load input window

    - Member Edit Mode (future feature)

      - Edit length, edit spans, edit bearing length in viewer

    - Add hole (future feature)

      - Need to create member hole rules and logic

  - Right click on member bearing opens menu

    - Properties

      - Opens bearing properties window

    - Linked supports (future feature)

      - Opens window showing member(s) linked to this bearing for load transfer

  - Right click on load opens menu

    - Edit load

      - Opens load window for editing

    - Copy load

      - Copies the selected load and opens load window for editing

    - Delete load

      - Deletes the selected load

  - Right click on bearing reaction text opens menu (text showing bearing reactions after analysis)

    - Copy Reaction

      - Copies the reaction to be pasted onto another member

        - Once pasted the reaction is linked to the member it was copied from. If the member from the copied reaction is updated, the reaction auto updates (future feature, part of the Linked supports feature).

- Member Design Window (below member viewer window)

  - Design Tabs top of member design window)

    - Span

      - Shows span information inputs

    - Settings

      - Shows member design input settings

    - Loads

      - Shows loading buttons and inputs

    - Products

      - Shows available products from user material library

    - Holes

      - Shows hole design inputs & settings

    - Notch / Bevel

      - Shows end modification inputs & settings

    - Notes

      - Shows text inputs for user notes

  - Member Design Window (changes based on which design tab is selected)

    - Span tab

      - Number of spans input with + and  -

      - Span Type dropdown

        - Out to Out (default)

          - Dimension from outside of bearing to outside of bearing

          - Uses center of bearing for interior bearings if multiple spans

        - Clear Span

          - Dimension from inside of bearing to inside of bearing

          - Uses center of bearing for interior bearings if multiple spans

        - Center to Center

          - Dimension from center of bearing to center of bearing

          - Uses center of bearing for interior bearings if multiple spans

      - Enable Bearing Analysis

        - Checkbox (default to on)

        - Will need to create bearing analysis engine

      - Span input

        - One box for 1 span (default)

        - Increases inputs based on number of spans

      - Cantilever checkbox

        - Only for end spans

      - Support Label Input

        - One per bearing

        - Defaults to B1, B2, B3, B4, and so on

        - Can be manually changed

      - Support Type dropdown

        - Wall / Plate (default)

          - Default

          - Shows bearing plate for member support in member viewer window

        - Beam

          - Shows dropped beam for member support in member viewer window

        - Column

          - Shows column for member support in member viewer window

        - Hanger

          - Shows hanger for member support in member viewer window

      - Support Material dropdown (for bearing analysis)

        - Steel

        - Douglas Fir

        - Southern Pine

        - Spruce-Pine-Fir

        - LVL 1.5E - 1.7E

        - LVL 1.8E - 2.3E

        - Unspecified (default)

        - Will add more once everything is working

      - Support Width

        - Default to 3-½”

        - Every bearing has an input box for bearing width

        - We can add default bearing widths in a dropdown

    - Settings tab

      - Tributary Width for Area Loads input box

      - Max Depth checkbox

        - Allows you to input the max depth for member

      - Service Condition

        - Dry (default)

        - Wet

      - Usage

        - Dropped Beam

        - Flush Beam (default)

        - Wall Header

      - Lateral Torsion Buckling

        - Member Bracing Type - Top

          - Braced at All Supports

          - Braced at Member Ends

          - Fully Braced

          - Specify Bracing Length

            - Input box appears to add length

        - Member Bracing Type - Bottom

          - Braced at All Supports

          - Braced at Member Ends

          - Fully Braced

          - Specify Bracing Length

            - Input box appears to add length

      - Live Load Deflection

        - Fixed Live Load Deflection

          - L/360 (default)

        - Cantilever Span Deflection

          - L/360 (default)

      - Total Load Deflection

        - Fixed Total Load Deflection

          - L/240 (default)

        - Cantilever Load Deflection

          - L/240 (default)

        - Max Total Deflection

          - 1” (default)

        - Max Cantilever Deflection

          - 1” (default)

    - Loads

      - Load Input Buttons (along top of window below tabs)

        - Add Uniform Load

          - Opens uniform load input window

        - Add Uniform Linear Load

          - Opens uniform linear load input window

        - Add Trapezoidal Linear Load

          - Opens trapezoidal linear load input window

        - Add Point Load

          - Opens point load input window

        - Add Concentrated Linear Load

          - Opens concentrated linear load input window

      - Crud Buttons (next to load input buttons)

        - Copy

          - Copy selected load

            - Opens load edit window to edit copied load

        - Edit

          - Edit selected load

            - Opens load edit window for selected load

        - Delete

          - Deletes selected load

        - Delete All

          - Deletes all loads

      - Load List Window

        - Load List Headers

          - Load Tag

            - 1, 2, 3, 4, etc.

          - Description

            - Load description

          - Load Type

            - Uniform Load

            - Uniform Linear Load

            - Trapezoidal Linear Load

            - Point Load

            - Concentrated Linear Load

          - Reference

            - Left

              - Inputs load using dimension from left side of member

            - Right

              - Inputs load using dimension from right side of member

          - Start

            - Load starting point from reference point

          - End

            - Load ending point from reference point

          - Location

            - Top

              - Load applies to top of member

            - Flush

              - Load applies to side of member

          - Tributary Width

            - Tributary width of uniform area load

          - Load Type

            - Live Load

            - Dead Load

            - Snow Load

            - Roof Load

            - Wind Load

          - Load Value

            - Numerical load values from load input window

            - lbs/ft2

            - lbs/ft

            - lbs

          - Load Value

            - 100% - LL

            - 90% - DL

            - 115% - SL

            - 125% - RL

            - 160% - WL

        - Load List

          - Blank until user inputs loads

          - Shows values that go with header list once loads are input

    - Products

      - Product List Headers

        - Material / Series

          - Member material and series

          - Modifies ply width and depth based on selected materials

        - Plies

          - 1, 2, 3 or 4

        - Width

          - Material Ply Width

            - Shows width list of selected materials

        - Depth

          - Material Depth

            - Shows depth list of selected materials

        - Product

          - Product list of available materials to analyze based on material, ply, width and depth selections

    - Holes

      - Buttons across top of window

        - Add

          - Add a hole

            - Opens hole input window

        - Edit (grayed out until hole is added)

          - Edit selected hole

            - Opens hole edit window

        - Copy (grayed out until hole is added)

          - Copy selected hole

            - Opens hole edit window for copied hole

        - Delete (grayed out until hole is added)

          - Delete selected hole

        - Delete All

          - Deletes all holes

      - Hole List Headers

        - Tag

          - 1, 2, 3, 4, etc

        - Description

          - Hole description

        - Center

          - Dimension of center of hole based on horizontal reference point

        - Reference

          - From left to right

          - From right to left

        - Elevation

          - Dimension of center of hole based on vertical reference point

        - Reference

          - From bottom to top

          - From top to bottom

        - Height

          - Height of hole

        - Width

          - Width of hole

        - Shape

          - Shape of hole

            - Circular

            - Rectangle

            - Obround

            - Rounded Rectangular

        - Orientation

          - Horizontal

            - Through face of member

          - Vertical

            - Through depth of member

    - Notch / Bevel

      - Shows both left and right end of member

      - Type Box (one for left and one for right)

        - None

          - Default

        - Notch

          - Location

            - Top of member

            - Bottom of member

          - Depth

            - Vertical depth of notch from reference location

          - Length

            - Horizontal length of notch from end of member

        - Bevel Cut

          - Location

            - Top only

          - Heel Depth

            - Vertical height of end of member after cut

          - Bevel Slope

            - Pitch of bevell cut #/12

    - Connections (grayed out if single ply)

      - Multiple Ply tab

        - Fastener Preference (if condition requires bolts or screws over nails, then only bolts or nails are shown)

          - Bolts

          - Nails (default)

          - Screws

        - Fastener Manufacturer (if nails are selected as connector, they will still show no matter what manufacturer is selected)

          - Simpson Strong-Tie Inc.

            - Shows list of only Simpson products

              - Screws or bolts

          - Mitek Structural Connectors

            - Shows list of only Mitek products

              - Screws or bolts

          - Fastenmaster

            - Shows list of only Fastenmaster products

              - Screws or bolts

        - Connector List Header

          - Description

            - Connector description (ex. 16d Box Nails)

          - OCS

            - On center spacing of the connectors

          - Rows

            - Number of rows of connectors

          - Capacity

            - Capacity of connectors in lbs/ft based on connector type, ocs and rows

          - Load

            - Load required to support in lbs/ft based on member analysis

          - Allowable

            - The % of the required load to support to the capacity of the connectors (Load / Capacity * 100)%

          - Duration

            - Duration of the loads

        - Connector List

          - Alphabetical list of available connectors

            - Only shows connectors that work to support the load

            - Only shows connector type and manufacturer that is selected

              - Unless nails is default and they don’t work

      - Concentrated Loads tab (grayed out if no flush concentrated loads applied to member)

        - Load Number dropdown

          - All loads

            - Applies connection to all concentrated side loads

          - Load # (1, 2, 3, etc.)

            - Applies connection to a specific concentrated side load

        - Fastener Preference (if condition requires bolts or screws over nails, then only bolts or nails are shown)

          - Bolts

          - Nails (default)

          - Screws

        - Fastener Manufacturer (if nails are selected as connector, they will still show no matter what manufacturer is selected)

          - Simpson Strong-Tie Inc.

            - Shows list of only Simpson products

              - Screws or bolts

          - Mitek Structural Connectors

            - Shows list of only Mitek products

              - Screws or bolts

          - Fastenmaster

            - Shows list of only Fastenmaster products

              - Screws or bolts

        - Connector List Header

          - Description

            - Connector description (ex. 16d Box Nails)

          - Quantity

            - Quantity of connectors required

          - Capacity

            - Capacity of connectors in lbs based on material type and quantity

          - Load

            - Load required to support in lbs based on applied side load

          - Allowable

            - The % of the required load to support to the capacity of the connectors (Load / Capacity * 100)%

          - Duration

            - Duration of the loads

        - Connector List

          - Alphabetical list of available connectors

            - Only shows connectors that work to support the load

            - Only shows connector type and manufacturer that is selected

              - Unless nails is default and they don’t work

    - Notes

      - Member Description

        - Input box for member description to show on member report

      - Notes

        - Input box to add notes to the member report

      - Notes Library (future feature)

        - Library of pre-loaded or manually written notes to show on the member report

  - Member Analysis Window (always shows to the right of member design window)

    - Analysis Header List

      - Control Type

        - Moment

          - Moment of analysed member

        - Negative Moment

          - Negative moment of analysed member

        - End Shear

          - End shear of analysed member

        - Continuous Shear

          - Continuous shear of analysed member

          - Only shows if there is continuous shear

            - Multiple span members

        - Total Load Deflection

          - Total load deflection of analysed member

        - Live Load Deflection

          - Live load deflection of analysed member

        - Total Negative Deflection

          - Total negative deflection of analysed member

          - Only shows if there is total negative deflection

            - Multiple span members

        - Max Deflection

          - Max deflection of analysed member

        - Cantilever Max Deflection

          - Max deflection of cantilever of analysed member

          - Only shows if there is a cantilever

        - Span / Depth Ratio

          - Span to depth ratio of analysed member

        - Member Bearings

          - Member bearings of analysed member

          - Shows all bearings (B1, B2, B3, etc.)

        - Uplift

          - Uplift at bearing of analysed member

          - Only shows if there is uplift at a bearing

      - Control Value

        - Max value of the control type for each control type of the analysed member

      - Control %

        - % of max control value for each control type

    - Blank if member analysis hasn’t been run